### 1. Input Signal (Vin)

Where to Measure:

- Measure at the point where the audio signal enters the circuit, before the PWM stage.

What to Observe:

- A low-amplitude sine wave or analog signal corresponding to your audio input (e.g., from a phone or signal generator).

- Amplitude depends on your source (e.g., 100 mV to 1 V peak-to-peak).

### 2. After Signal Conditioning (if applicable)

Where to Measure:

- At the output of the LM358 (or any op-amp used for buffering or amplifying the input signal).

What to Observe:

- A sine wave with increased amplitude (if the op-amp is used for gain).

- The offset voltage may be visible here, aligning the signal within the comparator’s range.

### 3. PWM Signal Generation

Where to Measure:

- At the output of the LM358 (or comparator), which generates the PWM signal.

What to Observe:

- A square wave with varying duty cycle corresponding to the amplitude of the input signal.

- The frequency of the PWM signal should remain stable (e.g., 50 kHz), while the duty cycle changes based on the audio input.

### 4. Gate Drive Signal (MOSFET Gate)

Where to Measure:

- Measure at the gate terminals of the MOSFETs.

What to Observe:

- A high-frequency PWM signal with varying duty cycle (similar to the comparator output).

- Ensure the signal has sufficient amplitude to fully switch the MOSFETs (e.g., 10–15 V for most MOSFETs).

### 5. Drain-Source Voltage (MOSFET Output)

Where to Measure:

- Measure at the drain or source of the MOSFET (depending on the configuration).

What to Observe:

- A high-frequency, amplified PWM signal.

- This will be a higher-voltage version of the PWM signal seen at the gate.

### 6. Filter Output (Reconstructed Signal)

Where to Measure:

- At the output of the low-pass filter, before it reaches the speaker.

What to Observe:

- A smooth, amplified sine wave corresponding to the original audio signal.

- The amplitude should match the amplified output voltage (e.g., several volts peak-to-peak depending on the design).

### 7. Speaker Output Voltage

Where to Measure:

- Across the terminals of the speaker.

What to Observe:

- A sine wave that drives the speaker to produce sound.

- Its amplitude depends on the gain of the amplifier and the power delivered.

### 8. Power Supply Voltage

Where to Measure:

- At the input to the power supply pins of the LM358, MOSFETs, and other components.

What to Observe:

- A stable DC voltage that powers the circuit.

- For example, VCCV_{CC} might be +12 V or +24 V.

### 9. Offset Voltage

Where to Measure:

- At the input to the comparator or after the offset adjustment potentiometer.

What to Observe:

- A DC bias added to the input signal to align it with the comparator’s range.

- For example, if the comparator’s input range is 0–5 V, the offset might be around 2.5 V.

### Key Instruments Needed:

- Oscilloscope: To observe waveforms at various stages.

- Multimeter: To measure DC and AC voltage levels.

- Function Generator: (Optional) To provide a test audio signal if needed.